Western Australia

Road Traffic (Vehicles) Act 2012

# Road Traffic (Vehicles) Amendment Regulations (No. 3) 2024

# Western Australia

Road Traffic (Vehicles) Amendment Regulations (No. 3) 2024

Contents

1.	Citation	1

2.	Commencement	1

3.	Regulations amended	1

4.	Regulation 171 amended	1

5.	Regulation 180 amended	3

Road Traffic (Vehicles) Act 2012

Road Traffic (Vehicles) Amendment Regulations (No. 3) 2024

Made by the Governor in Executive Council.

##### 1.	Citation

These regulations are the Road Traffic (Vehicles) Amendment Regulations (No. 3) 2024.

##### 2.	Commencement

These regulations come into operation as follows —

(a)	regulations 1 and 2 — on the day on which these regulations are published on the WA legislation website;

(b)	the rest of the regulations — on the day after that day.

##### 3.	Regulations amended

These regulations amend the Road Traffic (Vehicles) Regulations 2014.

##### 4.	Regulation 171 amended

Delete regulation 171(1) and insert:

(1A)	In this regulation —

safer freight vehicle means a heavy vehicle, other than a trailer or bus, that —

(a)	meets the requirements of the following ADRs, so far as they are applicable —

(i)	ADR 14/03 or a later version of ADR 14;

(ii)	ADR 35/07 or a later version of ADR 35;

(iii)	ADR 97;

(iv)	ADR 99;

and

(b)	if the vehicle has a GVM of more than 8 t — meets the requirements of ADR 105, so far as it is applicable; and

(c)	if the vehicle is not a prime mover —

(i)	meets the requirements of ADR 106; and

(ii)	is fitted with conspicuity markings that comply with the requirements of ADR 13.

(1)	A motor vehicle, trailer or any vehicle in a combination must not be over —

(a)	if the vehicle is a safer freight vehicle — 2.55 m wide; or

(b)	otherwise — 2.5 m wide.

##### 5.	Regulation 180 amended

In regulation 180(2) delete:

Vehicle loaded to width limits mentioned in r. 170, 
with maximum side projection of load allowed under r. 180

and insert:

Maximum width limit of vehicle and its load set under r. 171(1)(b), with maximum side projection of load allowed under r. 180

K. COLLERAN, Clerk of the Executive Council

© State of Western Australia 2024.

This work is licensed under a Creative Commons Attribution 4.0 International Licence (CC BY 4.0). To view relevant information and for a link to a copy of the licence, visit www.legislation.wa.gov.au.

Attribute work as: © State of Western Australia 2024.

By Authority: GEOFF O. LAWN, Government Printer

Endnotes

[For ease of reference detach these notes and read them alongside the draft.]